Art No. 273

PN40 Brass HVAC Valve

Combo Strainer Valve

Features

- Working Medium: Water,

- Pipe thread to ANSI B1.20.1 NPT/ ISO228 /ISO7

- Solder Socket to ANSI B16.18

- A Combination Ball Valve, Strainer and Union

- Fixed end connection in FNPT or SWT or G or Rc

- Tail piece connection in MNPT,FNPT and SWT or G or Rc,Size Reduction Available

- Standard Features include adjustable Memory Stop and Pressure/Temperature Port, Plug and Drain Valve

202 IS

Technical  Data

- Nominal Pressure：600WOG

Working Temperature：-20℃～+110℃；

Material specifications

| No. | Part Name | Materials |
| --- | --- | --- |
|  | Body | Brass CW617N or DZR CZ132 |
|  | Fixed End | Brass CW617N or DZR CZ132 |
|  | Ball | Brass or DZR CZ132 |
|  | Seat | PTFE |
|  | Stem | Brass CW617N or DZR CZ132 |
|  | O-ring | EPDM |
|  | Tail Piece | Brass CW617N or DZR CZ132 |
|  | Union Nut | Brass CW617N |
|  | Handle | Carbon Steel or Stainless steel 304 |
|  | Hex Nut | Carbon Steel or Stainless steel 304 |
|  | Memory Stop | Carbon Steel |
|  | Drain Valve | Brass CW617N |
|  | PT Valve | Brass CW617N |
|  | Strainer | S.S.304, 20Mesh |

Dimensions for reference  (in mm)

| Size | 1/2” | 3/4” | 1” | 1 1/4” | 1 1/2” | 2” |
| --- | --- | --- | --- | --- | --- | --- |
| L(FNPT) | 106 | 126 | 130 | 168.5 | 169 | 232 |
| H | 50.3 | 53 | 53 | 62 | 62 | 79 |
| D | 93 | 93 | 93 | 128 | 128 | 144 |
| B | 15 | 20 | 25 | 32 | 40 | 49 |
| E | 15.93 | 19 | 24 | 35 | 41.35 | 54.1 |